Used UTV/ATV Pre-Purchase Quick Checklist

Simple items a buyer can check before purchasing a used side-by-side, UTV, or ATV

This checklist is for easy visual and functional checks only. For higher-dollar, turbo, modified, high-hour, or questionable machines, schedule a professional inspection before buying.

Machine Information

| Year/Make: ____________________ | Model: ____________________ | VIN: ____________________ | Miles/Hours: ____________________ |
| --- | --- | --- | --- |
| Asking Price: ____________________ | Seller: ____________________ | Location: ____________________ | Date: ____________________ |

Paperwork / Ownership

| Check | Item | Notes / Concern |
| --- | --- | --- |
| [ ] | VIN on machine matches title, bill of sale, or registration paperwork |  |
| [ ] | Seller name matches paperwork or seller can explain ownership chain |  |
| [ ] | No obvious VIN tampering, ground-off numbers, or altered tags |  |
| [ ] | Seller can explain service history, modifications, and known issues |  |

Walkaround / Frame / Body

| Check | Item | Notes / Concern |
| --- | --- | --- |
| [ ] | Machine sits level and does not look twisted, bent, or sagged |  |
| [ ] | Cage, frame, bumpers, doors, and skid plates look straight |  |
| [ ] | No obvious fresh paint, undercoating, or pressure-washing hiding damage |  |
| [ ] | No major missing bolts, broken tabs, loose panels, or zip-tie repairs |  |
| [ ] | No strong fuel, coolant, burnt oil, belt, or electrical smell |  |

Fluids / Leaks

| Check | Item | Notes / Concern |
| --- | --- | --- |
| [ ] | Engine oil level looks correct and is not milky, glittery, or burnt-smelling |  |
| [ ] | Coolant bottle has coolant and no oily sludge or heavy rust color |  |
| [ ] | No active leaks under the machine after sitting and after running |  |
| [ ] | No heavy oil residue around engine, transmission, front diff, or rear gearcase |  |
| [ ] | Air filter and air box are clean with no signs of dust passing the filter |  |

Tires / Wheels / Brakes

| Check | Item | Notes / Concern |
| --- | --- | --- |
| [ ] | Tires are same size left-to-right and have usable tread |  |
| [ ] | No major sidewall damage, plugs everywhere, or bent/cracked wheels |  |
| [ ] | No missing lug nuts or broken wheel studs |  |
| [ ] | Brake pedal feels firm and machine stops straight |  |
| [ ] | No grinding noise, fluid leak, or obvious brake damage |  |

Suspension / Steering

| Check | Item | Notes / Concern |
| --- | --- | --- |
| [ ] | Steering turns smoothly with no major clunks, binds, or excessive play |  |
| [ ] | Wheels do not appear tilted, pushed back, or uneven side-to-side |  |
| [ ] | No obviously bent arms, radius rods, trailing arms, or tie rods |  |
| [ ] | Shocks are not leaking oil and springs are not broken or collapsed |  |
| [ ] | No loud clunks when rocking the machine side-to-side |  |

Engine Start / Dash / Electrical

| Check | Item | Notes / Concern |
| --- | --- | --- |
| [ ] | Starts cold without excessive cranking, smoke, or abnormal knocking |  |
| [ ] | Idles smoothly after startup and responds cleanly to light throttle |  |
| [ ] | No check engine light, limp mode, overheating light, or warning messages |  |
| [ ] | Headlights, brake lights, dash, winch, and accessories work if equipped |  |
| [ ] | Battery does not sound weak during cranking |  |

4WD / Drive Mode Function Check

| Check | Item | Notes / Concern |
| --- | --- | --- |
| [ ] | 2WD and 4WD engage and disengage correctly; dash indicator matches selection |  |
| [ ] | In 4WD, front tires pull under light load; no front diff/axle popping or clicking |  |
| [ ] | Differential lock, turf mode, or sport/work modes function if equipped |  |
| [ ] | No binding, banging, grinding, delayed engagement, or flashing drive-mode warnings |  |

Test Drive - Safe Area Only

| Check | Item | Notes / Concern |
| --- | --- | --- |
| [ ] | Warm the machine fully before judging performance |  |
| [ ] | Shifts into park, reverse, neutral, low, and high correctly |  |
| [ ] | Steers straight and brakes straight without pulling, wobble, or vibration |  |
| [ ] | Temperature stays normal during the drive and fan cycles normally if equipped |  |
| [ ] | With seller permission, perform a controlled full-throttle / "hammer-down" pull in a safe legal area |  |
| [ ] | Under full load, no misfire, hesitation, belt squeal, clutch slip, transmission grind, driveline vibration, smoke, or power loss |  |

Buyer Red Flags

| Check | Item | Notes / Concern |
| --- | --- | --- |
| [ ] | Seller refuses title/VIN verification or will not allow a reasonable test drive |  |
| [ ] | Machine is already warm before you arrive and seller avoids a cold start |  |
| [ ] | Freshly pressure-washed with no service records or clear maintenance history |  |
| [ ] | Overheating, smoke, misfire, knocking, slipping belt, or warning lights |  |
| [ ] | Signs of rollover, flood/submersion, dusted engine, or major frame damage |  |
| [ ] | Seller says "it just needs a belt" but it will not drive correctly under load |  |

Decision

[ ] Looks Good    [ ] Needs Professional Inspection    [ ] Negotiate Price    [ ] Walk Away

| Notes | ___________________________________________________________________________ |
| --- | --- |
| Questions for Seller | ___________________________________________________________________________ |

Before you buy someone else's problem, download our free UTV/ATV Pre-Purchase Inspection Checklist - or schedule a professional inspection with San Tan SxS Repair for extra peace of mind.